一、SPI驱动框架

1.1 裸机下的SPI驱动框架

首先编写SPI控制器驱动，bsp_spi.c和bsp_spi.h。最终提供SPI读写函数：spich0_readwrite_byte

spi具体芯片驱动：ICM20608

1.2、Linux下的SPI驱动框架

主机控制器驱动：SOC的SPI外设驱动，此驱动是半导体原厂编写好的，为spi-imx.c，当spi控制器的设备和驱动匹配以后，spi_imx_probe函数就会执行，完善I.MX6ULL的SPI控制器驱动。

SPI控制器驱动核心就是spi_master的构建，spi_master里面就有如何通过SPI控制器与SPI外设进行通信的函数，此函数是原厂编写的。

spi_master->transfer

spi_master-> transfer_one_message	6ULL主机控制器使用此函数，

设备驱动：具体的SPI芯片驱动

1.3、SPI主机驱动

1、spi_master

struct spi_master {

struct device	dev;

struct list_head list;

/* other than negative (== assign one dynamically), bus_num is fully

* board-specific.  usually that simplifies to being SOC-specific.

* example:  one SOC has three SPI controllers, numbered 0..2,

* and one board's schematics might show it using SPI-2.  software

* would normally use bus_num=2 for that controller.

*/

s16			bus_num;

/* chipselects will be integral to many controllers; some others

* might use board-specific GPIOs.

*/

u16			num_chipselect;

/* some SPI controllers pose alignment requirements on DMAable

* buffers; let protocol drivers know about these requirements.

*/

u16			dma_alignment;

/* spi_device.mode flags understood by this controller driver */

u16			mode_bits;

/* bitmask of supported bits_per_word for transfers */

u32			bits_per_word_mask;

#define SPI_BPW_MASK(bits) BIT((bits) - 1)

#define SPI_BIT_MASK(bits) (((bits) == 32) ? ~0U : (BIT(bits) - 1))

#define SPI_BPW_RANGE_MASK(min, max) (SPI_BIT_MASK(max) - SPI_BIT_MASK(min - 1))

/* limits on transfer speed */

u32			min_speed_hz;

u32			max_speed_hz;

/* other constraints relevant to this driver */

u16			flags;

#define SPI_MASTER_HALF_DUPLEX	BIT(0)		/* can't do full duplex */

#define SPI_MASTER_NO_RX	BIT(1)		/* can't do buffer read */

#define SPI_MASTER_NO_TX	BIT(2)		/* can't do buffer write */

#define SPI_MASTER_MUST_RX      BIT(3)		/* requires rx */

#define SPI_MASTER_MUST_TX      BIT(4)		/* requires tx */

/* lock and mutex for SPI bus locking */

spinlock_t		bus_lock_spinlock;

struct mutex		bus_lock_mutex;

/* flag indicating that the SPI bus is locked for exclusive use */

bool			bus_lock_flag;

/* Setup mode and clock, etc (spi driver may call many times).

*

* IMPORTANT:  this may be called when transfers to another

* device are active.  DO NOT UPDATE SHARED REGISTERS in ways

* which could break those transfers.

*/

int			(*setup)(struct spi_device *spi);

/* bidirectional bulk transfers

*

* + The transfer() method may not sleep; its main role is

*   just to add the message to the queue.

* + For now there's no remove-from-queue operation, or

*   any other request management

* + To a given spi_device, message queueing is pure fifo

*

* + The master's main job is to process its message queue,

*   selecting a chip then transferring data

* + If there are multiple spi_device children, the i/o queue

*   arbitration algorithm is unspecified (round robin, fifo,

*   priority, reservations, preemption, etc)

*

* + Chipselect stays active during the entire message

*   (unless modified by spi_transfer.cs_change != 0).

* + The message transfers use clock and SPI mode parameters

*   previously established by setup() for this device

*/

int			(*transfer)(struct spi_device *spi,

struct spi_message *mesg);

/* called on release() to free memory provided by spi_master */

void			(*cleanup)(struct spi_device *spi);

/*

* Used to enable core support for DMA handling, if can_dma()

* exists and returns true then the transfer will be mapped

* prior to transfer_one() being called.  The driver should

* not modify or store xfer and dma_tx and dma_rx must be set

* while the device is prepared.

*/

bool			(*can_dma)(struct spi_master *master,

struct spi_device *spi,

struct spi_transfer *xfer);

/*

* These hooks are for drivers that want to use the generic

* master transfer queueing mechanism. If these are used, the

* transfer() function above must NOT be specified by the driver.

* Over time we expect SPI drivers to be phased over to this API.

*/

bool				queued;

struct kthread_worker		kworker;

struct task_struct		*kworker_task;

struct kthread_work		pump_messages;

spinlock_t			queue_lock;

struct list_head		queue;

struct spi_message		*cur_msg;

bool				idling;

bool				busy;

bool				running;

bool				rt;

bool				auto_runtime_pm;

bool                            cur_msg_prepared;

bool				cur_msg_mapped;

struct completion               xfer_completion;

size_t				max_dma_len;

int (*prepare_transfer_hardware)(struct spi_master *master);

int (*transfer_one_message)(struct spi_master *master,

struct spi_message *mesg);

int (*unprepare_transfer_hardware)(struct spi_master *master);

int (*prepare_message)(struct spi_master *master,

struct spi_message *message);

int (*unprepare_message)(struct spi_master *master,

struct spi_message *message);

/*

* These hooks are for drivers that use a generic implementation

* of transfer_one_message() provied by the core.

*/

void (*set_cs)(struct spi_device *spi, bool enable);

int (*transfer_one)(struct spi_master *master, struct spi_device *spi,

struct spi_transfer *transfer);

void (*handle_err)(struct spi_master *master,

struct spi_message *message);

/* gpio chip select */

int			*cs_gpios;

/* DMA channels for use with core dmaengine helpers */

struct dma_chan		*dma_tx;

struct dma_chan		*dma_rx;

/* dummy data for full duplex devices */

void			*dummy_rx;

void			*dummy_tx;

};

2、spi_imx_data

struct spi_imx_data {

struct spi_bitbang bitbang;

struct completion xfer_done;

void __iomem *base;

struct clk *clk_per;

struct clk *clk_ipg;

unsigned long spi_clk;

unsigned int count;

void (*tx)(struct spi_imx_data *);

void (*rx)(struct spi_imx_data *);

void *rx_buf;

const void *tx_buf;

unsigned int txfifo; /* number of words pushed in tx FIFO */

/* DMA */

unsigned int dma_is_inited;

unsigned int dma_finished;

bool usedma;

u32 rx_wml;

u32 tx_wml;

u32 rxt_wml;

struct completion dma_rx_completion;

struct completion dma_tx_completion;

struct dma_slave_config rx_config;

struct dma_slave_config tx_config;

const struct spi_imx_devtype_data *devtype_data;

int chipselect[0];

};

3、spi_bitbang

struct spi_bitbang {

spinlock_t		lock;

u8			busy;

u8			use_dma;

u8			flags;		/* extra spi->mode support */

struct spi_master	*master;

/* setup_transfer() changes clock and/or wordsize to match settings

* for this transfer; zeroes restore defaults from spi_device.

*/

int	(*setup_transfer)(struct spi_device *spi,

struct spi_transfer *t);

void	(*chipselect)(struct spi_device *spi, int is_on);

#define	BITBANG_CS_ACTIVE	1	/* normally nCS, active low */

#define	BITBANG_CS_INACTIVE	0

/* txrx_bufs() may handle dma mapping for transfers that don't

* already have one (transfer.{tx,rx}_dma is zero), or use PIO

*/

int	(*txrx_bufs)(struct spi_device *spi, struct spi_transfer *t);

/* txrx_word[SPI_MODE_*]() just looks like a shift register */

u32	(*txrx_word[4])(struct spi_device *spi,

unsigned nsecs,

u32 word, u8 bits);

};

spi_imx_data-> bitbang-> master

-> setup_transfer

-> txrx_bufs

spi_master，一般需要申请spi_alloc_master，释放的话就是spi_master_put。

需要初始化spi_master，最终使用函数spi_register_master注册。

bitbang下的spi_imx_setupxfer函数

spi_imx_setupxfer-> spi_imx->rx = spi_imx_buf_rx_u8;		//最终的SPI接收函数

->spi_imx->tx = spi_imx_buf_tx_u8;		//发送函数

->spi_imx->devtype_data->config(spi_imx, &config);-> mx51_ecspi_config		//配置6ULL的SPI控制器寄存器

bitbang下的txrx_bufs，示例是spi_imx_transfer函数：

spi_imx_transfer

->spi_imx_pio_transfer

->spi_imx_push(spi_imx)

->spi_imx->tx(spi_imx)

最终调用：spi_bitbang_start，

->master->transfer_one_message = spi_bitbang_transfer_one;

->spi_register_master,向系统注册spi_master。

spi_bitbang_transfer_on

-> bitbang->txrx_bufs(spi, t)= spi_imx_transfer

-> spi_imx->tx(spi_imx)= spi_imx_buf_tx_u8.

所以，经过这么复杂的操作，最终目的就是，设置spi_master的transfer_one_message函数为spi_imx_buf_tx_u8。

4、收发函数

spi_imx->rx和spi_imx->tx，以一个字节为例：

spi_imx->rx = spi_imx_buf_rx_u8;

spi_imx->tx = spi_imx_buf_tx_u8;

5、SPI接收

SPI通过中断接收，中断处理函数为spi_imx_isr，此函数会调用spi_imx->rx(spi_imx)函数来完成具体的接收过程。

1.4、SPI设备驱动

SPI设备驱动就是具体的SPI芯片驱动，比如ICM20608。

spi_device：每个spi_device下都有一个spi_master。每个SPI设备，肯定挂载到了一个SPI控制器，比如ICM20608挂载到了6ULL的ECSPI3接口上。

spi_driver：非常重要！本讲重点，申请或者 定义一个spi_driver，然后初始化spi_driver中的各个成员变量，当SPI设备和驱动匹配以后，spi_driver下的probe函数就会执行！

spi_driver初始化成功以后需要向内核注册，函数为：spi_register_driver，当注销驱动的时候需要spi_unregister_driver

二、驱动编写与测试

1、修改设备树，添加IO相关信息

ECSPI3_SCLK		->	UART2_RXD

ECSPI3_MOSI		->	UART2_CTS

ECSPI3_SS0		-> 	UART2_TXD

ECSPI3_MISO		->	UART2_RTS

片选信号不作为硬件片选，而是作为普通的GPIO，我们在程序里面自行控制片选引脚。

2、在ECSPI3节点下创建icm20608子节点

3、需要初始化icm20608芯片、然后从里面读取原始数据！这个过程就要用到如何使用linux内的SPI驱动API来读写ICM20608

用到两个重要的结构体：spi_transfer和spi_message

spi_transfer用来构建收发数据内容。

构建spi_transfer，然后将其打包到spi_message里面，需要使用spi_message_init初始化spi_message，然后在使用spi_message_add_tail将spi_transfer添加到spi_message里面，最终使用spi_sync和spi_async来发送。